A new software, S/W 2.1, has been introduced in Beomster 4500 for functional purposes with BL 5000/7000.

At the same time, the following faults have been corrected:

1. Beosystem 4500 may intermittently block and it is necessary to remove the mains plug.
2. If a two-figure track is selected on tape or CD, the second figure may disappear.
3. AM long wave has been changed from 155 kHz to 153 kHz, lowest frequency.
4. The STORE button does not function in test mode/service mode on local operation.
5. Search-tuning has been improved for a better STEREO/MONO switch.
The modifications have been introduced in the production from serial no. 08499434. The part number is the same, 8341178.

A new software version 2.2 has been introduced in Beomaster 4500 as from serial no. 08819263.

The following modifications have been made:
1. The fault in the PLAY NEXT function has been corrected.
2. Beomaster 4500 does not transmit IR status when option 0 mode is selected.

The new software version 2.2 can be ordered under the usual part no.: 8341178.

Symptom:

When Beomater 4500 is connected to AV 9000 via audio kit, it is impossible to record satellite sound on the audio tape recorder.

Furthermore, with the LIGHT programming ON early Sunday evening and OFF Sunday evening at 23.30 hours, the light turns off, but is on again Monday morning.

Solution:

Replace S/W with service S/W, part 8342407.

The S/W has not been implemented in the production.

Improved IR transmit/receive facility (two-way communication) of Beomaster 4500 when the unit is hanging on the wall.

Besides having an IR transmitter/receiver situated on the front panel the Beomaster 4500 has now been improved by the installation of an additional IR transmitter/receiver that operateds through the display glass.
This additional IR transmitter/receiver has been introduced from serial no. 08682954.
On a service basis the improvement can also be obtained by replacing the following components:
- Operation PCB (module 06) 8001106
- IR receiver PCB (module 05) 8001392
- Display glass (pos. 9011) 3168782

In units with a serial number higher than 08499834 it is not necessary to replace the display glass (pos. 9011).