Problem:

No picture.

Cause:

Flashes damage the power processor 01IL01 (leak on pin 7).

Solution:

Remove diode 01OL76.
Mount a 15 volts zener diode, part no. 8300053, between pins 6 and 7 on 01IL01. Mount the zener diode on the copper foil side of PCB 01, cathode against pin 7.

The solution will be introduced in the production.

A new type of power processor 01IL01 has been introduced. The old type is TEA 2026 T, and the new one is TEA 2026 C.

When replacing the power processor, the new type can substitute the old one directly.

In connection with the introduction of the new type of power processor the following components have been removed:

01DP18
01DP19
01DP20
01DP22
01CL82
01RP23

These components must of course be mounted, if the power processor is of the old type.